MSDS No: 79-00-5 Date: 03/09/2001

| SUPPLIER ADDRESS: | 6141 Easton Road, Bldg. 1 PO Box 310 Plumsteadville, PA 18949-0310 | EMERGENCY PHONE NUMBER: | (215) 766-8861 |
| --- | --- | --- | --- |

1. CHEMICAL PRODUCT

| PRODUCT NAME: | 1,1,2-TRICHLOROETHANE | SYNONYMS: | Ethane Trichloride, B-Trichloroethane, Vinyl Trichloride |
| --- | --- | --- | --- |

2. COMPOSITION, INFORMATION ON INGREDIENTS

|  | Exposure Limits (PPM) |
| --- | --- |

| Ingredient Name | Formula |  | CAS # |  | Concentration |  | ACGIH TLV |  | OSHA PEL |  | MAC |  | Other STEL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,1,2-TRICHLOROETHANE | C2H3CL3 |  | 79-00-5 |  | 99+% |  | 10 |  | 10 |  | 10 |  | NE |

| Note: | NE = NONE ESTABLISHED | S/A = SIMPLE ASPHYXIANT |
| --- | --- | --- |

3. HAZARD INDENTIFICATION

* * * EMERGENCY OVERVIEW * * *
Flammable liquid and vapor.
Can form explosive mixtures with air.
May cause central nervous system damage.
Can cause damage to eyes, skin, and respiratory tract.

POTENTIAL HEALTH EFFECTS

ROUTES OF ENTRY: Inhalation , Skin , Ingestion 

ACUTE EFFECTS: Vapor exposure can cause irritation of the eyes, nose, throat and respiratory tract. Skin exposure may cause irritation, itching, erythema, swelling, burning and pain. Eye contact may cause irritation, redness, or blurred vision. Possible central nervous system depression. Symptoms include shortness of breath, headache, confusion, nausea, dizziness, and unconsciousness. Can be absorbed through the skin. 

CHRONIC EFFECTS: Kidney and liver damage. 

MEDICAL CONDITIONS AGGRAVATED BY OVEREXPOSURE: Respiratory problems. 

OTHER EFFECTS OF OVEREXPORSURE: None

CARCINOGENICITY (US ONLY):

NTP - No
IARC MONOGRAPHS - No
OSHA REGULATED - No

4. FIRST AID MEASURES

INHALATION: Immediately remove victim to fresh air. If breathing has stopped, give artificial respiration. If breathing is difficult, give oxygen. 

EYE CONTACT: Immediately flush with copious amounts of water for at least 15 minutes. Do not allow victim to rub or keep eyes tightly shut. 

SKIN CONTACT: Immediately flush with copious amounts of water for at least 15 minutes while removing contaminated clothing. 

INGESTION: Never give anything by mouth to an unconscious person. Have conscious and alert person drink 1 to 2 glasses of water. 

IN EVENT OF EXPOSURE, CONSULT A PHYSICIAN

NOTE TO PHYSICIAN: None

5. FIRE FIGHTING MEASURES

FLASH POINT: N/Av

AUTOIGNITION TEMPERATURE: 460 deg. C

FLAMMABLE LIMITS: Vol. %

LOWER: 8.4
UPPER: 13.3

EXTINGUISHING MEDIA: Extinguish with water spray, water fog, dry chemical, or carbon dioxide. 

SPECIAL FIRE FIGHTING INSTRUCTION AND EQUIPMENT: Wear self-contained breathing apparatus and full protective clothing. Keep fire exposed cylinders cool with water spray. 

HAZARDOUS COMBUSTION PRODUCTS: Toxic carbon monoxide, hydrogen chloride and phosgene. 

UNUSUAL FIRE AND EXPLOSION HAZARDS: Cylinder rupture may occur under fire conditions. Emits toxic fumes under fire conditions. Vapors may travel a considerable distance to the source of ignition and flash back.

6. ACCIDENTAL RELEASE MEASURES

CLEAN UP PROCEDURES: Evacuate and ventilate area. Remove leaking cylinder to exhaust hood or safe outdoor area. Shut off source if possible and remove source of heat. Absorb with sand or vermiculite and place in closed containers for disposal. 

SPECIALIZED EQUIPMENT: None

7. HANDLING AND STORAGE

PRECAUTIONS TO BE TAKEN IN HANDLING: Secure cylinder when using to protect from falling. Use suitable hand truck to move cylinders. Use only in a well-ventilated area. 

PRECAUTIONS TO BE TAKEN IN STORAGE: Store in well ventilated areas. Keep valve protection cap on cylinders when not in use. Store away from oxidizers, combustible materials, and source of ignition or heat.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

ENGINEERING CONTROLS: Provide adequate general and local exhaust ventilation to maintain concentrations below exposure and flammable limits. 

EYE / FACE PROTECTION: Safety glasses 

SKIN PROTECTION: Impervious gloves, coveralls, boots, and/or other resistant protective clothing. 

RESPIRATORY PROTECTION: Use a self-contained breathing apparatus in case of emergency or non-routine use. 

OTHER PROTECTIVE EQUIPMENT: Safety shoes when handling cylinders.

9. PHYSICAL AND CHEMICAL PROPERTIES

APPEARANCE: Colorless 

ODOR: Sweet odor. 

PHYSICAL PRESSURE: Liquid 

VAPOR PRESSURE: @35 deg. C: 40 mm Hg

VAPOR DENSITY (AIR=1): 4.6

BOILING POINT (C): 114

SOLUBILITY IN WATER: Slight 

SPECIFIC GRAVITY (H2O=1): @20 deg. C: 1.44

EVAPORATION RATE: N/Av

ODOR THRESHOLD: N/Av

10. STABILITY AND REACTIVITY

STABILITY: Stable under normal storage conditions. 

CONDITIONS TO AVOID: Storage in poorly ventilated areas. Storage near a heat source. 

MATERIALS TO AVOID: Potassium, powdered aluminum, magnesium, zinc, oxidizing agents, amines and amides. 

HAZARDOUS POLYMERIZATION: Will not occur. 

HAZARDOUS DECOMPOSITION: HCl gas, phosgene gas, CO and oxides of chlorine.

11. TOXICOLOGICAL INFORMATION

LETHAL CONCENTRATION (LC50): 4051 ppm, Rat 1 hour.

LETHAL DOSE 50 (LD50): N/Ap

TERATOGENICITY: N/Ap

REPRODUCTIVE EFFECTS: N/Ap

MUTAGENICITY: N/Ap

12. ECOLOGICAL INFORMATION

No adverse ecological effects are expected.

13. DISPOSAL CONSIDERATIONS

WASTE DISPOSAL METHOD: Dispose of non-refillable cylinders in accordance with federal, state and local regulations. Allow gas to vent slowly to atmosphere in an unconfined area or exhaust hood. If the cylinders are the refillable type, return cylinders to supplier with any valve outlet plugs or caps secured and valve protection caps in place.

14. TRANSPORT INFORMATION

CONCENTRATION: 99+%

DOT DESCRIPTION (US ONLY):

PROPER SHIPPING NAME: Flammable liquid, n.o.s.
HAZARD CLASS: 3 (flammable), Packing Group 1
INDENTIFICATION NUMBER: UN1993
REPORTABLE QUANTITIES: 100 lbs.
LABELING: FLAMMABLE LIQUID

ADR / RID (EU Only): Class 3, 1A

SPECIAL PRECAUTIONS: Cylinders should be transported in a secure upright position in a well ventilated truck.

15. REGULATORY INFORMATION

OSHA: Process Safety Management: Material is not listed in appendix A of 29 CFR 1910.119 as highly hazardous chemical. 

TSCA: Material is listed in TSCA inventory. 

SARA: The threshold planning quantity for material is 10,000 lbs. 

EU NUMBER: 201-166-9

NUMBER IN ANNEX 1 OF DIR 67/548: Not listed in annex 1. 

EU CLASSIFICATION: N/Av

R: 20-21-22

S: 9

16. OTHER INFORMATION

OTHER PRECAUTIONS: Protect containers from physical damage. Do not deface cylinders or labels. Cylinders should be refilled by qualified producers of compressed gas. Shipment of a compressed gas cylinder which has not been filled by the owner or with his written consent is a violation of federal law (49 CFR). 

ABBREVIATIONS: N/Ap - Not Applicable N/Av - Not Available SA - Simple Asphyxiant NE - None Established 

DISCLAIMER: Information included in this document is given to the best of our knowledge, however, no warranty is made that the information is accurate or complete. We do not accept any responsibility for damages by the use of the document.